Anleitung zum Laden von Firmware in die CUVC

1.	Anlegen eines Verzeichnisses C:\VCBOOT (MD VCBOOT)

In das Verzeichnis C:\VCBOOT wechseln (CD VCBOOT)

Datei VC_Vxxx.EXE nach C:\VCBOOT kopieren

Datei VC_Vxxx.EXE ausfuehren, es werden die erforderlichen Dateien

ausgepackt

2.	Verbindung von PC und CUVC mit Kabel laut Kompendium Kapitel 7.3.1

herstellen. Es kann dasselbe Kabel wie beim MC verwendet werden.

3.	Den MASTER DRIVES mit Spannung versorgen. Die PMU muss EIN 'E' an-

zeigen

4.	Aus dem Verzeichnis heraus, in dem sich nun die entpackten Files be-

finden nun die Batchdatei 'bslvc.bat' aufrufen.

Fuer Win2000 bitte WinBsl.exe starten.

5. 	Nach erfolgreichem Firmware-Download meldet sich die PMU mit 'o000' .

6.	Bitte stellen Sie sicher, zur Firmwareversion jeweils das zugeh�rige

Kompendium zu verwenden.

Diese Dokumente finden Sie auf der dem Umrichter beigepackten CD.

Bemerkung: Wenn ein Firmwareupdate mithilfe eines USB-RS232-Umsetzers

durchgef�hrt wird, so kann eine geringere Baudrate erforderlich sein.

(bei WinBSL.exe Knopf "Schnittstelle" � "Baudrate": 38400 oder 19200).

How to load firmware into the CUVC

1.	Make directory C:\VCBOOT (MD VCBOOT)

Change to directory C:\VCBOOT (CD VCBOOT)

Copy file VC_Vxxx.EXE to C:\VCBOOT

Start VC_Vxxx.EXE, this will unpack all files

2.	Connect the CUVC and your PC with a cable described in the compendium

chapter 7.3.1. The cable for MC firmware download can be used for CUVC,

too.

3. 	Switch on the power supply for the MASTER DRIVES. The PMU must display

ONE 'E'.

4.	Start the batch file 'bslvc.bat' in the temporary directory.

For Win2000 start WinBsl.exe.

5.	After a successful download the PMU displays 'o000'.

6.	Please use the Compendium supplied with the firmware Version.

You will find these documents on the CD packed in with the converter.

Note: If a firmware update is carried out via a USB-RS232 converter,

then a lower baud rate can be required.

(WinBSL.exe button "interface" - "baud rate": 38400 or 19200).

Prinzipschaltbild des Verbindungskabels / Schematic of cable

PC				MASTER DRIVES

SUB-D 9-Pin Buchse/female	SUB-D 9-Pin Stecker/male

Pin 1

Pin 2 ------------------------- Pin 7

Pin 3 ------------------------- Pin 2

Pin 4

Pin 5 ------------------------- Pin 4, Pin 9, Pin 1, Pin 5

Pin 6

Pin 7

Pin 8

Pin 9